# Building a MicroBlaze System Using the EDK

Version 0.3 EDK3.1EA Build C10.5

Building a MicroBlaze System Using the EDK

## Introduction

This lab will illustrate the Embedded Development Kit (EDK) flow. The lab design is partially completed. You will complete the MHS file, C application code, and linker script. You will also correct an application code error.

## Objectives

After completing this lab, you will be able to:

- Design a hardware processor system including the MicroBlaze™ soft processor core and the associated IP

- Design a software application used to exercise the processor system

- Modify the software application and quickly update the hardware bit file using Data2BRAM

- Debug the software application using GNU Debugger (GDB) and Xilinx Microprocessor Debugger (XMD)

## Design Description

When designing any embedded processor system you need the following items:

- Required hardware

- Memory map of the system

- And the software application

This EDK lab example consists of the following hardware:

- MicroBlaze soft processor core

- LMB Bus

- LMB_LMB_bram_if_cntlr

- BRAM_BLOCK

- OPB Bus

- opb_gpio

- opb_bram_if_cntlr

- OPB BRAM

- opb_uartlite

LMB_LMB_BRAM_IF_CNTLR

LMB BRAM

MB

OPB_GPIO

OPB_GPIO

OPB_UARTLITE

OPB_BRAM_IF_CNTLR

OPB BRAM

OPB Bus

ILMB Bus

DLMB Bus

Figure 3-1. Design Layout.

- Below is the memory map for this design

| - Device | - Address |  | - Size | - Comment |
| --- | --- | --- | --- | --- |
|  | - Min | - Max |  |  |
| - LMB_BRAM | - 0x0000_0000 | - 0x0000_1FFF | - 16kB | - LMB Memory |
| - OPB_GPIO | - 0xFFFF_4200 | - 0xFFFF_42FF | - 256B | - DIP Switch Input |
| - OPB_GPIO | - 0xFFFF_4100 | - 0xFFFF_41FF | - 256B | - LED Output |
| - OPB_UARTLITE | - 0xFFFF_4000 | - 0xFFFF_40FF | - 256B | - Serial Output |
| - OPB_BRAM | - 0xFFFF_0000 | - 0xFFFF_3FFF | - 16kB | - OPB Memory |

- The provided lab files are incomplete. Through the course of the lab, you will complete the MHS file, the linker script, and the software application.

## Procedure

This lab comprises four primary sections: you will create the MicroBlaze system hardware platform, build the application software for the MicroBlaze system, download the bitstream to the FPGA, and finally debug the application software.  Below each general instruction for a given procedure, you will find accompanying step-by-step directions and illustrated figures providing more detail for performing the general instruction. If you feel confident about a specific instruction, feel free to skip the step-by-step directions and move on to the next general instruction in the procedure.

I. 	Creating the MicroBlaze System Hardware Platform

#### Create an XPS Project	Step 1

The Xilinx Platform Studio allows you to control the hardware and software development. It also provides an editor and a project management interface to create and edit source code. The XPS offers software tool flow configuration options. XPS also creates a Project Navigator project allowing you control of the hardware implementation flow in a familiar environment.

XPS supports the creation of the MSS file (Microprocessor Software Specification), the MVS file (Microprocessor Verification Specification), and software tool flows associated with this software specification. It supports customization of software libraries, drivers, interrupt handlers and the compilation of the user program using the EDK default Linker Script or by providing a Custom one.

- Open XPS: StartProgramsXilinx Embedded Development KitXilinx Platform Studio

- In XPS, select FileNew Project

- Create New Project dialog box opens as shown in figure 3-2.

Figure 3-2. Create new Project dialog box.

- Use the Project File Browse button to browse to the C:\training\microblaze\labs\mainlab folder. Click Open to create the system.xmp file, as shown in figure 3-3

- Figure 3-3. XPS Project Files Directory.

- Use the MHS File to import Browse button to select your system.mhs file

- Select the system.mhs file and click Open as shown in figure 3-4

- Figure 3-4. Adding the MHS file to the XPS Project.

- Set the Target Device to the following:

- Architecture: Virtex2

- Device Size: xc2v1000

- Package: fg456

- Speed Grade: -4

- Click OK to create the project

#### Hardware Entry: ADD IP	Step 2

The Microprocessor Hardware Specification (MHS) file defines the hardware component of the design. An MHS file defines the configuration of the embedded processor system, and includes the following:

- Bus architecture

- Peripherals

- Processor

- Connectivity of the system

- Interrupt request priorities

- Address space

In this design, the lmb_lmb_bram_if_cntlr with the associated BRAM and the OPB UART have not been included in the MHS file. You will need to add this IP to finish the hardware design. XPS provides a utility to easily add the peripheral definition to a new or existing MHS file. The user is then required to specify the correct parameters and port connections.

- Double click on system.mhs in XPS to open it

- Examine the mhs file. Notice the section headers for the missing IP

- Close the system.mhs file

- Select ProjectAdd Cores to open the Cores List dialog. (see figure 3-5)

- Figure 3-5.

- Select the opb_uartlite_v1_00_b and click Add to MHS

- Select the lmb_lmb_bram_if_cntlr_v1_00_a and click Add to MHS

- Select the bram_block_v1_00_a and click Add to MHS

- Click the “x” to close the Cores List dialog

Connect and configure IP.

- Double click on system.mhs in XPS to open it

- Move each of the peripherals to the appropriate place

- Properly configure the C_BASEADDR and C_HIGHADDR for the lmb_lmb_bram_if_cntlr and the opb_uartlite using the MEMORY MAP table

- Complete the lmb_lmb_bram_if_cntrl to match the following:

- BEGIN lmb_lmb_bram_if_cntlr

- # Generics for vhdl or parameters for verilog

- PARAMETER INSTANCE = inst_lmb_lmb_bram_if_cntlr

- PARAMETER HW_VER = 1.00.a

- PARAMETER C_MASK = 0x00800000

- PARAMETER C_BASEADDR = 0x00000000

- PARAMETER C_HIGHADDR = 0x00001fff

- # Global ports

- PORT LMB_Clk = sys_clk

- # Bus Interfaces

- BUS_INTERFACE ILMB = i_lmb

- BUS_INTERFACE DLMB = d_lmb

- BUS_INTERFACE PORTA = lmb_porta

- BUS_INTERFACE PORTB = lmb_portb

- END

- The BUS_INTERFACE ILMB = i_lmb connects the slave i_lmb bus to the MB peripheral.

- The BUS_INTERFACE PORTA = lmb_porta connects the BRAM to this controller.

- Complete the bram_clock for use by the lmb_lmb_bram_if_cntlr to match the following:

- BEGIN bram_block

- PARAMETER INSTANCE = bram1

- PARAMETER HW_VER = 1.00.a

- BUS_INTERFACE PORTA = lmb_porta

- BUS_INTERFACE PORTB = lmb_portb

- END

- You can see that the lmb_porta is used as the bus interface between the bram_block and the  lmb_lmb_bram_if_cntlr block.

- Complete the opb_uartlite to match the following:

- ### OPB UARTLite

- BEGIN opb_uartlite

- PARAMETER INSTANCE = myuart

- PARAMETER HW_VER = 1.00.b

- # Generics for vhdl or parameters for verilog

- PARAMETER C_HIGHADDR   = 0xFFFF40FF

- PARAMETER C_BASEADDR   = 0xFFFF4000

- PARAMETER C_DATA_BITS  = 8

- PARAMETER C_CLK_FREQ   = 100000000

- PARAMETER C_BAUDRATE   = 9600

- PARAMETER C_USE_PARITY = 0

- # Global ports

- #PORT OPB_Rst = sys_rst

- PORT OPB_Clk = sys_clk

- BUS_INTERFACE SOPB = myopb

- # uart signals

- PORT RX = rx

- PORT TX = tx

- END

- You can see that the BUS_INTERFACE SOPB = myopb connects the opb_uartlite to the OPB bus. You can also see that there are several PARAMETERs used to configure the uartlite for communication with the hyperterminal of your host.

- In order for the OPB_UARTLITE to communicate with the outside world, these signals need to be in the global port list. Make sure that the following ports are in the global ports list

- PORT RX = rx, DIR = INPUT

- PORT TX = tx, DIR = OUTPUT

- Save and close the file

- In XPS select ProjectResync Project to reload the mhs file

- You should now see the new IP being displayed in XPS.

- To add additional IP/hardware in the MHS file:

- There are two ways to add additional logic into the FPGA design. One method is to include additional IP/hardware into a processor system using the MHS file. The second method is to instantiate the processor system in a top level wrapper file. This second method will be discussed later.

- To incorporate additional IP/hardware into a processor system a strict directory structure must be maintained.

- Refer to figure 3-6 for a depiction of the peripheral directory structure.

$XIL_MYPERIHPERALS

drivers

myip

user_ip

user_ip

hdl

netlist

simmodels

data

verilog

vhdl

MPDD

BBDD

PAOD

- Figure 3-6. Peripheral Directory Structure

- The user is responsible for creating this structure and the associated files.

- To specify additional directories, use one of the following options:

- Current directory (where Platform Generator was launched; not where the MHS resides)

- Set the Platform Generator -P option, or the XIL_MYPERIPHERALS environment variable

- Platform Generator uses a search priority mechanism to locate peripherals, as follows:

- 1. Search current directory in the myip directory

- 2. Search $XIL_MYPERIPHERALS/myip (UNIX) or %XIL_MYPERIPHERALS%\myip (PC)

- 3. Search $XILINX_EDK/hw/coregen/ip/xilinx/pcores*/com/xilinx/ip2/processor (UNIX) or %XILINX_EDK%\hw\coregen\ip\xilinx\pcores*\com\xilinx\ip2\processor (PC)

- The first two search methods (1 and 2) have the same underlying directory structure and should be used for user IP. The third search area has the CORE Generator directory structure and should not be modified by the user. This information has been included for your information only.

#### Generate a Netlist and Create a Project	Step 3

Now that the hardware has been completely specified in the MHS file, Platform Generator (platgen) can be run. Platgen elaborates the MHS file into a hardware system consisting of NGC files representing the processor system.

- In XPS, select ToolsGenerate Netlist

- The following directories will be created:

- implementation

- hdl

- synthesis

- xst

- Note: On the first run of Platgen, the netlist generation can take 15 minutes.  However, further runs shall be a lot quicker, due to cache operations by Platgen

- Specify the Design Hierarchy and Implementation Tool Flow. Select OptionsProject Options

- In the Project Options dialog, as shown in figure 3-7, select the Hierarchy and Flow tab

- Figure 3-7. The Set Project Options.

- Configure the Hierarchy and Flow tab with the following settings:

- Design Hierarchy: This is the top level of my design

- Netlist Generation: Hierarchical

- Synthesis Tool: None

- Implementation Tool Flow: ISE (ProjNav)

- NPL File: click the “…” button.

- This will open the “ProjNav Project (NPL) Files” dialog as shown in figure 3-8. In this dialog, create a new directory called proj_nav_proj in the root XPS project directory. Select this directory and click Open.

Figure 3-8. ProjNav Project (NPL) Files.

- Note: You want to make sure that the Project Navigator project is created in the root directory so that it does not get deleted when you clean up the XPS project.

- Click OK

- In XPS, select ToolsExport to ProjNav

- This will create a Project Navigator project in the directory previously specified.

- Open the Project Navigator project in ISE 5.1.01i

#### Implement the Design	Step 4

The Project Navigator project created by XPS does not contain all of the information necessary to implement the design. For example, UCF file must be added to the project. However, this flow allows you to add additional logic to the FPGA design using ISE.

- Select ProjectAdd Source to add the system.ucf file in the data directory

- Select system.vhd in the Source Window (don’t double-click yet)

- Select ‘Generate Programming File’ in the Process Window

- Select ProcessProperties

- This will open the Process Properties for the Generate Programming File process as shown in figure 3-9.

Figure 3-9. Generate Programming File Process Properties.

- Select the Startup options, and ensure that the FPGA Start-Up Clock is set to JTAG Clock

- This ensures the FPGA starts up correctly when programmed via a JTAG cable.

- Click Ok

- Select system.vhd

- Double click Generate Programming File in the Process Window

II.	Building the Application Software for the MicroBlaze System

#### Design Software	Step 1

Now that the hardware design has been completed, you will work on the software design.  If you closed XPS, then you need to reopen it and load the project located in the C:\training\microblaze\labs\mainlab directory.

You now need to get the bitstream (without programming information) from ISE back into XPS.

- Select ToolsImport from ProjNav

- Find the .bit file (system.bit) from the Project directory – proj_nav_proj

- Select the .bmm file that is output from ISE

- ISE input ‘bram_init.bmm’, and annotated this file through implementation.  The resulting output from bitgen should be bram_init_bd.bmm, in the EDK project implementation directory.

#### Specify the Driver Interface Levels	Step 2

For each of the peripherals being utilized in the design, you need to specify the driver interface level.

- In XPS, double click myuart

- The Peripheral Options dialog opens as shown in figure 3-10. This dialog contains Peripheral Attributes, Interrupt Handler Function, and Driver Configuration fields. It should be noted that there are two levels of drivers for each peripheral.

- Figure 3-10. Peripheral Options Dialog.

- The architecture of the device drivers is designed as a layered architecture as shown in Figure 3-11. The layered architecture accommodates the many use cases of device drivers while at the same time providing portability across operating systems, toolsets, and processors. The layered architecture provides seamless integration with an RTOS (Layer 2), high-level device drivers that are full-featured and portable across operating systems and processors (Layer 1), and low-level drivers for simple use cases (Layer 0). The following paragraphs describe each of the layers. You can choose to use any and all layers.

Figure 3-11. Drivers Layered Architecture

- Set the Interface Level to Level 1

- Click OK

- Set the Interface Level to Level 1 for the following peripherals:

- Inst_microblaze

- mygpin

- mygpout

- Note: For information regarding what functions are available for the different driver levels refer to the xilinx_driver.pdf located in the Xilinx_EDK\doc\xilinx_driver_api directory.

#### STDIN/STDOUT	Step 3

- Libgen provides you with the ability map printf, scanf, etc. to an I/O peripheral in your design.

- Double click inst_microblaze to open the “microblaze instance inst_microblaze” dialog

- Select the Processor Property tab

- In the Communication Peripherals section, use the pull-down for the STDIN and STDOUT Peripheral to select myuart

- Click OK

#### Develop Code	Step 4

For the purpose of this lab, a sample C program has been created. You will be responsible for fixing an error in the program during the debug section.

- Click instance_microblaze

- Select ProjectAdd Program Sources

- Add Source and Header Files open to the current processor dialog.

- Select the system.c file located in the C:\training\microblaze\labs\mainlab\code directory

- Double click system.c to open it in XPS

- Examine the contents to make sure you understand the functionality.

#### Compile Code	Step 5

- Using the GNU GCC Compiler, compile the application code and BSP.

- In XPS, select ToolsGenerate Libraries

- Libgen runs and compiles the drivers associated with this design.

- Select ToolsCompile Program Sources

- mb-gcc runs and compiles the source files.

?

- Question 1: How big is the code in the elf file? __________________________

- Let's find out.

- Open a Xygwin shell (Use the YG Button from the Tool Bar Menu)

- In the Xygwin shell cd to the project directory

- cd to the inst_microblaze/code directory

- Enter the following command: mb-size executable.elf

- This will show you the size of the various sections of the elf file.

- Now we can look at a memory dump of the object file:

- Enter the following command: mb-objdump –s executable.elf > hexdump.txt

- This shall output a HEX dump of the object code to the text file

- Open the text file created in step 7, above, in Wordpad

?

- Question 2: At what address has the application code been placed? (.text)

- _________________________________________________

- Question 3: At what address has the ReadOnly Data been placed? (.rodata) _________________________________________________

- Question 4: At what address has the Small Data been placed ? (.sdata2) _________________________________________________

- Question 5: At what address has the Data been placed ? (.data) _________________________________________________

- Question 6: What is the address range used by this ELF file?

- _______________________ to ________________________

- Question 7: Does the MicroBlaze system have memory completely covering this range? _____________________

- Hexdump.txt should show that the ELF file cannot fit in the Blockmemory specified on the LMB.  One solution would be to increase this memory.  However, we shall use a linker script to push some of this data into the OPB block RAM.

- Close hexdump.txt

#### Create Linker Scripts	Step 6

- To tell the GNU linker where to place the code, a linker script is required. A linker script has been provided with this design.

- In XPS, select inst_microblaze

- Select ProjectAdd Program Sources

- In the Add Source and Header Files to the current processor dialog, change the “Files of type:” to “All Files(*.*)”

- Select mapfile located in the code directory and click Open

- Note: Adding the mapfile to the project does not cause XPS to utilize it.

- Double click mapfile to open it in XPS

?

- Question 8: Examine the contents. In which memory region will the .text section be placed?

- _____________________________________________________________

- Question 9: How about the _heap and _stack ? _____________________________________________________________

- Question 10: How about .rodata ? _____________________________________________________________

- Question 11: How about the .sdata2 and .data regions ? _____________________________________________________________

- Close the mapfile

- Open the microblaze instance inst_microblaze dialog.

- Select OptionsSet OptionsCompiler Options to open the microblaze instance inst_microblaze dialog

- Select the Directories tab

- In the Linker Options section, select the mapfile located in the code directory as the Linker Script

- Click OK

- In XPS, select ToolsCompile Program Sources

- This will run mb-gcc which compiles the source files.

- Enter the following command: mb-objdump –s executable.elf > hexdump.txt

- This shall output a HEX dump of the object code to the text file.

- Open the file created in step 6, above, in Wordpad

?

- Question 12: At what address has the application code now been placed? ___________________________________________________________

- Question 13: How about the data section ? __________________________________________________________

- Question 14: What address ranges does the ELF file now use ?

- ____________to ______________ and

- ____________to ______________

- Question 15: Does the MicroBlaze system have memory completely covering this range? __________________

- The use of the linker script allows you to change where portions of code, and data are stored.  This could allow you to put executable, or modifiable data into external RAM, and also readonly data into FLASH / ROM.

- III.  Downloading the Bitstream to the FPGA

#### Download and Configure the FPGA	Step 1

- Now that the hardware and software design has been completed, the device can be configured. In this section you will download and configure the FGPA.

- Connect the host computer to the target board

- This includes connecting the Parallel 3/4 cable and the serial cable.

- Start a hyperterminal session with the following settings:

- com1

- Bits per second : 9600

- Data bits: 8

- Parity: none

- Stop bits: 1

- Flow control: none

- Turn the board power on

- Turn all of the DIP switches on except number 8

- In XPS, select ToolsUpdate Bitstream and Download

- This will create a new bit file, which has been updated with the recently compiled code. Impact will then be used to configure the device.

- Once the device has been configured the hyperterminal should output a message.  Press any key on the terminal to continue.  The terminal should now look similar to figure 3-12.

- Figure 3-12. Hyperterminal Output

- As the message states the DIP switches control the rate at which the counter counts. By turning on switch two the count is now delayed by three seconds. The delay can be specific from 0 to 255 in binary using all eight switches.

?

- Question 16: If you turn on the second and third DIP switch, is the count delayed by seven seconds? _____________________________________

- Question 17: Does the HyperTerminal DIP output equal seven? ________________________

- IV.  Debugging the Application Software

#### Find Coding Errors	Step 1

It is now your job to find this software coding error!!!

With the device configured, the software application can be debugged directly via the xmd_stub software. GDB connects to the xmd_stub core through a utility called the Xilinx Microprocessor Debug (XMD) Engine. XMD is a program that facilitates a unified GDB interface as well as a Tcl (Tool Command Language) interface for debugging programs and verifying systems using the MicroBlaze or PowerPC (Virtex-II Pro) microprocessors.

The XMD Engine is used along with MicroBlaze and PowerPC GDB (mb-gdb & powerpc-eabi-gdb) for debugging. mb-gdb and powerpc-eabi-gdb communicates with xmd using the Remote TCP protocol and control the corresponding targets. In either case, GDB can connect to xmd on the same computer or on a remote computer on the Internet. Figure 3-13 illustrates the connections.

mb-gdb

MicroBlaze System

User Interface

GDB Remote Protocol

(TCP/IP)

TCL/Terminal Interface

XMD

cycle-accurate Instruct Set Simulator

UART

UART Lite

JTAG UART

XMD Stub

Built-in Simulator

Figure 3-13. GDB and XMD Connections.

#### Download Software	Step 2

- First, we need to download the XmdStub software to the MicroBlaze processor.

- Double-click the inst_microblaze component

- On the Processor Property tab, set the ‘Mode’ bullet to choose XmdStub mode

- Ensure that the Debug Peripheral is set to ‘myuart’  Press OK

- Close the hyperterminal

Download the XndStub firmware to the MicroBlaze system.

- Back in XPS, select ToolsUpdate bitstream and Download

- This shall download the XmdStub firmware to the MicroBlaze system.

- Open a XMD shell, by clicking the  icon

- After xmd has initialized, enter the command: mbconnect stub –comm serial –port COM1 –baud 9600

- This command shall attempt to connect to the Xmd-Stub now running in the hardware, via COM1 at 9600 baud.

- Take a note of the TCP port that the server is running on.  (This is usually 1234).

- In XPS, select ToolsSoftware Debugger

- The GDB GUI opens.

- In GDB, select FileTarget Settings

- The Target Selection dialog appears.

- Configure the Target Selection dialog to match figure 3-14.

- Figure 3-14. Target Selection dialog.

- Click Ok

Find code in GDB

- In GDB, select FileOpen

- Select executable.elf located in the inst_microblaze/code directory

?

- Question 18: Do you see the C code or the assembly code? _________________________________

- Question 19: Why can you not see the C code?

- __________________________________________________________________

#### Select Compile Options	Step 3

Select Compiler options

- In GDB, select FileExit

- In XPS, select OptionsCompiler Options

- In the microblaze instance inst_microblaze dialog, select the Optimization tab

- Check the “Create symbols for debugging (-g option)

- Click Ok

- Recompile the code and load the new executable elf into GDB.

?

- Question 20: Do you see the C code? _________________ If you do not see the C code, redo steps 1– 5, above.

- Select RunRun

- There is an automatic breakpoint at main. GDB allows the user to single step the C code or assembly code. Using these features find the software coding error, ask if you need help!!! This is an exercise to help you learn how to run GDB.

- Note: The default values displayed in the Registers Window are in hex, while the values displayed in the Source Window are in decimal.

- Once you have determined the error, recompile the code and download it through GDB.