# MicroBlaze Custom Peripheral Lab

Version 1.3 – EDK EA C10.5

MicroBlaze Custom Peripheral Lab

## Introduction

In this lab, a custom peripheral shall be added into a MicroBlaze™ system.  Both custom VHDL and C drivers shall be used, as well as the OPB common libraries.

## Objectives

After completing this lab, you will be able to:

- Integrate a custom peripheral into a MicroBlaze system consisting of both hardware (peripheral) and software (driver) components

## Design Description

- The design that is going to be built for this lab consists both of peripherals supplied with the Embedded Development Kit, and also a custom peripheral supplied as VHDL source code. This example uses a custom peripheral to perform the function of a 7-segment display driver.  The code is generic, and can support up to 10 seven-segment displays.

- The software drivers for this module contain a function that will input an integer number.  The driver interfaces to the peripheral hardware to display the number on the 7 segment displays in decimal form.

- The system consists of a UART, which is connected to the DOPB.  This UART receives data from an RS232 line, and the MicroBlaze system inputs data as it is received, using drivers supplied with the UART peripheral.  The number of characters input to the UART is counted by the MicroBlaze system, and periodically written to the drivers, controlling the 7-segment display.  The 7-segment display shall then display the number of characters that have been read from the UART in decimal form.

- Additionally, a button is connected to the MicroBlaze system through a GPIO peripheral.  This button shall reset the count output to the 7-segment display.

- Figure 5-1 illustrates the microprocessor layout for this lab.

Figure 5-1. microprocessor system layout illustrating bus architectures and external connections.

?

Question 1: Which Bus Structures are used by this application?

____________________________________________________________________________________________________________________________________________________________________________________________________________

Question 2: Which MicroBlaze configuration would most suit this application?

________________________________________________________________

________________________________________________________________

________________________________________________________________

## Procedure

This lab investigates the 7-segment peripheral.  Below each general instruction for a given procedure, you will find accompanying step-by-step directions and illustrated figures providing more detail for performing the general instruction. If you feel confident about a specific instruction, feel free to skip the step-by-step directions and move on to the next general instruction in the procedure.

The design files required for this lab are in the following directory: c:\training\microblaze\labs\custom\. The custom 7-segment peripheral is in the directory ‘custom_peripherals’ within the project directory.

?

Question 3: Where is and what is the name of the file that defines the signals and attributes of the 7-segment peripheral?

____________________________________________________________________________________________________________________________________________________________________________________________________________

Question 4: Which VHDL files are required for the peripheral to be built, and which file specifies this?

________________________________________________________________

________________________________________________________________

________________________________________________________________

Investigating the 7-Segment Peripheral

The 7-Segment peripheral uses four VHDL files:

OPB_7seg.vhd – This is the peripheral top level.  It deals with the OPB bus requirements and activates chipselects for the core.

Figure 5-2. Basic Structure of OPB_7Seg.vhd.

GPIO_core.vhd – This contains the logic required to register the data sent from the OPB.

Hex_to_7seg.vhd – This contains the logic to convert the stored 4-bit data into outputs suitable to drive a seven-segment display.

Pselect.vhd – This VHDL source comes with the embedded software and performs the function of an address decoder in Xilinx fast carry logic.  Pselect.vhd is synthesized for this peripheral, as it is included in the library declaration, in OPB_7SEG.vhd, and also in the .pao file.

Figure 5-3. Example Pselect implementation for decoding the address range 0xFFFF8000 to 0xFFFF803F.

Note: This is only an overview of the peripheral.  For more information about this peripheral, please refer to the commented source code.  Additionally, refer to the custom peripherals course module.

Drivers

This peripheral has a software driver in order to help the developer to use this peripheral.  This driver is located in the following directory:

custom_peripherals/drivers/drv_opb_7seg_v2_00_a/

src/7seg.h – This is a header file indicating the driver subroutines available to the user application

src/7seg.c – This file contains the source code in C for the driver.  In this peripheral, the driver is drv_7seg.  The function inputs an unsigned integer value and the address of the 7-segment peripheral.  The integer is separated into its decimal powers and each digit is written to the display at the appropriate memory address.

Src/makefile – This file controls how the source code is compiled and into which library it is placed.

Data/drv_opb_7seg.mdd – This file defined the drivers and which files Libgen should copy into the projects include directory.

Software

The software for this example is in the following location:

C:/training/microblaze/labs/custom/mymicroblaze/code/system.c

System.c is the user software, and performs the function as illustrated in Figure 4.  The count value is initialized, and set to zero.  The UART status registers are polled to see if there is new data in the UART FIFO.  If this is the case, the count is incremented, and the LEDs updated.  If the UART is empty, the GPIO is sampled for the pushbutton.  If the pushbutton is activated, the count value is reset, and the display updated.

Figure 5-4. Flowchart of the software: system c.

#### Launch the Xygwin Shell	Step 1

Launch the Xygwin shell from the START bar.

- Launch the Xygwin shell from the START bar:

- Locate the  directory where the lab files are stored, e.g.,  
cd c:/training/microblaze/labs/custom  ls

- Open the .mhs file in NOTEPAD: Notepad system.mhs

?

Question 5: What baud rates is the UART set to?

________________________________________

Question 6: What is the instance name of the 7-segment peripheral?

________________________________________________________________

________________________________________________________________

________________________________________________________________

Question 7: What is the address range of the Local Memory block RAMs, and how many Virtex-E block RAMs would this use?

________________________________________________________________

________________________________________________________________

________________________________________________________________

Question 8: Which peripheral needs to have drivers for the Standard IO (STDIO) input and output streams?

________________________________________________________________

________________________________________________________________

________________________________________________________________

- Close the system.mss file

We shall now run through the MicroBlaze command flow:

Figure 5-5. The MicroBlaze Command Flow.

We require platgen to create libraries targeting Virtex™-II specifying that the custom peripherals are in the custom_peripherals directory.

?

Question 9: What command should be run for libgen?

________________________________________________________________

________________________________________________________________

________________________________________________________________

- Run the command you specified in Question 9, above, to run libgen.

#### Compile 7-Segment Drivers	Step 2

To  confirm that this command has run successfully, the 7-segment drivers should now be compiled into the library along with the UART and GPIO drivers.

- The library is located in the ‘mymicroblaze/lib’ subdirectories within the project. Navigate to this directory: Cd mymicroblaze/lib

Within the directory, should be a file ‘libc.a’.  This is one of the libraries that mb-gcc shall use.  The functions for the 7-segment decoder should be in this library.

- Type the following command to check this:

Mb-objdump –a libc.a | grep 7seg

Mb-objdump outputs information about an object file, such as this library. For  more information about mb-objdump, consult the MicroBlaze documentation. Grep enables us to display just the data we are looking for. Information about grep can be found in any UNIX manual.

The result from this command should be similar to the following:

7seg.o:     file format elf32-microblaze

rw-r--r-- 500/513    884 Sep 7 13:51 2002 7seg.o

The output that can be seen is the entry for the 7-segment drivers in the project library.  As the 7-segment drivers are in this library, the C compiler shall be able to use these drivers in the user source code.

- If you do not see results similar to the above, please check your command for Libgen, or contact the lab tutor.

- Change back to the root of the project

Cd ..\..

You have now created the libraries for the compiler.  We now need to compile the code for executable mode specifying the Library directory to be ‘mymicroblaze/Lib’ and the Include library to be ‘mymicroblaze/Include’.

We need the executable file to be output to ‘mymicroblaze/code/system.elf’.  The input file is ‘mymicroblaze/code/system.c’.

?

Question 10: What command should be run for mb-gcc?

________________________________________________________________

________________________________________________________________

________________________________________________________________

The .elf file should now be available in the mymicroblaze/code directory.  Make sure that the compiler was set to executable mode.   This has completed the software part of the MicroBlaze EDK flow.  We now need to generate some hardware to run the code on.

We need to run Platgen targeting a Virtex-II device specifying the .mhs file that you looked at earlier.  We also need the peripheral directory set to be ‘custom_peripherals’.  Additionally, we want to set the synthesis tool to be Synplify.

?

Question 11: What command should be run for platgen?

________________________________________________________________

________________________________________________________________

________________________________________________________________

- Platgen should run through without any errors. If it cannot find the .mhs file, check to make sure that you are in the correct directory (c:/training/microblaze/labs/custom)

- The netlists have been generated for each of the peripherals in the ‘implementation’ directory.  Check to see if this is the case

- In the synthesis directory, there should be a synthesis project available for Synplify – System.prj

- Invoke Synplify from the Desktop icon

- Select FileOpen Project

- Navigate to the synthesis directory of the custom peripheral and load the system.prj file

#### Modify Project Properties	Step 3

It is required to modify the project properties to specify the demoboard settings.

- Click the target Change button and match them to the following figure:

Figure 5-6.

- Click OK to return to the main Synplify window

- Click the large RUN button

- The design should be implemented.

- If you are familiar with Synplify, check the RTL view to see what has been implemented.  You should be able to push down into the 7seg component to see its construction.

By now, you should have an EDIF netlist and implementation of your hardware.  Additionally, you should have an .elf file from the software flow. We shall use ISE to implement these and download to a demo board.

#### Implement and Download	Step 4

Use the ISE software tools to implement the files created in the steps, above, and download to a demo board.

- Launch ISE and create a new project

- Set the project directory to be “c:/training/microblaze/labs/custom/ise_project”

- Set the part type to be the same as that on the demo board (XC2V1000-FG456-4)

- Set the Flow to be EDIF

- Add the following files to the project:

C:/training/microblaze/labs/custom/implementation/system.edn

C:/training/microblaze/labs/custom/implementation/system.bmm

C:/training/microblaze/labs/custom/mymicroblaze/code/system.elf

C:/training/microblaze/labs/custom/data/system.ucf

The BMM file specifies the block RAMs that the executable code is going to be stored in.  The elf file shall then be inserted during bitgen to the correct block RAMs.

- Select the properties for implementation by right-clicking Implement design  Properties

- Set the macro search path to be C:/training/microblaze/labs/custom/implementation

This way, all of the required netlists shall be called in by the tools.

- Download to the board shall be via JTAG, so ensure that the JTAG clock is the startup clock in the ‘generate programming file’ startup properties

Now generate the programming file. The ISE tools should run through and the bitfile should be generated.  At the bitgen stage, the .elf file shall be included into the bitstream, so ensure that this has been included in the project.

Now load up iMPACT, with your board powered up and a programming cable connected between the board and your computer.

The chain should be detected and the bitstream downloaded.  Once completed, the done pin on the FPGA should go high and the 7-segment displays should indicate ‘00’.

An RS-232 cable should be connected between the board and your computer.  The board is looking for RS-232 communications from the PC.

To establish communications, load up Hyperterminal

- STARTProgramsAccessoriesCommunication Hyperterminal

- Give your session a name: e.g.,  ‘com1-19200 baud’

- Set up communications so that they match the following screenshot:

- Figure 5-7. Note the flow control is set to 'None'.

If this is all setup correctly, you should be able to type on the terminal, and the 7-segment display shall count the number of characters you send to it.

A

## Answers

- Which bus structures are used by this application?

Data OPB, Data LMB, Instruction LMB

- Which MicroBlaze configuration would most suit this application?

DOPB+DLMB+ILMB = Configuration 3.  This configuration can be found in the Hardware Reference Guide.

- Where is and what is the name of the file that defines the signals and attributes of the 7-segment peripheral?

Custom_peripherals/opb_peripherals/opb_7seg_v1_00_a/data/opb_7seg.mpd

- Which VHDL files are required for the peripheral to be built and which file specifies this?

Custom_peripherals/opb_peripherals/opb_7seg_v1_00_a/data/opb_7seg.pao requires the following files: 
common_types_pkg, pselect, gpio_core, hex_to_7seg, opb_7seg

- What baud rate is the UART set to?

19200 Baud

- What is the instance name of the 7 Segment peripheral?

My_7seg

- What is the address range of the Local Memory block RAMs, and how many Virtex-E block RAMs would this use?

0x00000000  0x00001FFF = 8192 8-bit locations

1x VirtexE Block RAM = 16384 bits

 4 Block RAMs are required

- Which peripheral needs to have drivers for the Standard IO (STDIO) input and output streams?

Myuart, the UART Lite peripheral

- What command should be run for Libgen?

Libgen –m executable –a virtex2 –P custom_peripherals system.mss

- What command should be run for mb-gcc?

Mb-gcc –xl-mode-executable –L./mymicroblaze/lib –I./mymicroblaze/include –o mymicroblaze/code/system.out system.c

- What command should be run for Platgen?

Platgen –a virtex2 –P custom_peripherals system.mhs

## Conclusion

Custom peripherals are not difficult to add to a project.  The generic abilities of VHDL are used to full advantage in the generation of peripherals to enable the decoding of addresses and also custom features of the peripheral.

Any peripheral that connects to the On-chip Peripheral Bus needs to conform to the specifications of the OPB bus specification.  This includes the stimulation of the XFER_ACK signal when the peripheral is written to or read from.  As the OPB is an or-gate structured bus, the output data bus must be set to logic ‘0’ when a peripheral read is not in progress.

The use of the –P switch in libgen and platgen allows the tools to recognize the location of custom peripherals.  Libgen uses this switch in order to compile the drivers.  Platgen uses this to synthesize or copy the HDL files for the peripheral into the user project.